| Start-Work Checks ¦ De-Isolation and Re-Energising |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## Start Work Authority: Confirm below are in place and verified prior to starting work |  |  |  |  |  |  |
| # Save Your Life Actions All exposed person(s) performing work must confirm each of the action items below |  |  |  | Person(s) Performing Work (initial) | Start-Work Verifier (initial) |  |
| I have confirmed: |  |  |  |  |  |  |
| 1 | Circuit, system, and/or equipment to be de-isolated/re-energized is the one identified in the isolation plan or drawing. |  |  |  |  |  |
| 2 | Isolation devices are removed per the isolation plan or drawing. |  |  |  |  |  |
| 3 | Any personnel affected by the re-energizing activity have been notified. |  |  |  |  |  |
| 4 | The circuit, system, and/or equipment has been re-energized per plan or drawing. |  |  |  |  |  |
| ## Stop and seek help if any of the above controls/safeguards are not in place |  |  |  |  |  |  |
| ## Role |  | ## Printed Name | ## Date |  |  |  |
| Start -Work Verifier |  |  |  |  |  |  |

| Save Your Life Actions ¦ De-Isolation and Re-Energising |  |  |
| --- | --- | --- |
| ## Start Work Authority: I will not start work until I confirm… |  |  |
| Actions |  | How to (examples) |
| 1 | Circuit, system, and/or equipment to be de-isolated/re-energized is the one identified in the isolation plan or drawing. | • Tags or markings identify the circuit, system and/or equipment to be de-isolated/re-energized. Note: Confirm circuit, system, and/or equipment is ready to be de-isolated and re-energized (e.g. all work has been completed) |
| 2 | Isolation devices are removed per the isolation plan or drawing. | • Physically inspect: • Work area to confirm all mechanical, process and electrical isolations were removed per plan. • Bleed and vent points to confirm they are closed/open per plan or procedure. |
| 3 | Any personnel affected by the re-energizing activity have been notified. | • Personnel in/near the work area or affected by the re-energizing of circuit, system, and/or equipment are aware it is ready to be put back in service. • Personnel are not in the Line of Fire of re-energized potential hazardous energy sources (e.g., electrical, pressure, hydraulic, mechanical etc) |
| 4 | The circuit, system, and/or equipment has been re-energized per plan or drawing. | • Circuit, system, and/or equipment has been re-energized per the isolation plan or system/equipment operating procedure. |