| Start-Work Checks ¦ Energised/Live Electrical System |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## Start Work Authority: Confirm below are in place and verified prior to starting work |  |  |  |  |  |  |
| # Save Your Life Actions All exposed person(s) performing work must confirm each of the action items below |  |  |  | Person(s) Performing Work (initial) | Start-Work Verifier (initial) |  |
| I have confirmed: |  |  |  |  |  |  |
| 1 | The authorised work scope has been reviewed and agreed to. |  |  |  |  |  |
| 2 | Circuit/equipment to be worked on is the one identified in the plan. |  |  |  |  |  |
| 3 | Personnel are wearing PPE rated for: • The electrical hazard • The electrical voltage prior to entering any access restricted area. |  |  |  |  |  |
| 4 | Restricted access zones have been identified and barricaded. |  |  |  |  |  |
| 5 | An electrical standby person is in place during work activities. If an electrical standby is not required, continue to the next step. |  |  |  |  |  |
| 6 | Communication plan with the electrical standby person has been agreed to. |  |  |  |  |  |
| 7 | The insulated tools and testing equipment is: • certified • inspected • free from damage • rated for the task |  |  |  |  |  |
| 8 | An emergency response plan is in place is ready to be used. |  |  |  |  |  |
| ## Stop and seek help if any of the above controls/safeguards are not in place |  |  |  |  |  |  |
| ## Role |  | ## Printed Name | ## Date |  |  |  |
| Start -Work Verifier |  |  |  |  |  |  |

| Save Your Life Actions ¦ Energised/Live Electrical System |  |  |
| --- | --- | --- |
| ## Start Work Authority: I will not start work until I confirm… |  |  |
| Actions |  | How to (examples) |
| 1 | The authorised work scope has been reviewed and agreed to. | • Review work scope per the approved work permit. • Discuss stop work considerations if work situation changes. |
| 2 | Circuit/equipment to be worked on is the one identified in the plan. | • Equipment to be worked on is correct using tag numbers or cable markings. |
| 3 | Personnel are wearing PPE rated for: • The electrical hazard • The electrical voltage prior to entering any access restricted area. | • Crew has knowledge of electrical hazards (e.g., voltage, single phase/three phase and arc flash). • Personnel are wearing PPE rated for the electrical voltage (i.e., arc flash) prior to entering the restricted approach boundary and it has been inspected and is free from damage. |
| 4 | Restricted access zones have been identified and barricaded. | • Restrict access to defined areas according to company policy and/or applicable reglatory requirements (e.g., NFPA 70E) • Barriers are in place to limit access to the work area • The work area is monitored to prevent unauthorized access |
| 5 | An electrical standby person is in place during work activities. If an electrical standby is not required, continue to the next step. | • Dedicated electrical person(s) is present at the work area according to company policy and/or applicable regulatory requirements (e.g., NFPA 70E) and their responsibilities include: • Monitor personnel entering the restricted area • Monitor the area for changing conditions • Initiate the emergency rescue response if needed |
| 6 | Communication plan with the electrical standby person has been agreed to. | • A communication plan has been discussed with qualified electricians • Communication plan has been agreed to and tested with the work crew. • Stop work signals • How to initiate emergency response plan |
| 7 | The insulated tools and testing equipment is: • certified • inspected • free from damage • rated for the task | • Insulating materials such as rubber matting, or screening are in place. • Only insulated tools that have been rated/certified for the equipment’s maximum voltage are used. • Conduct a voltage function test prior to using testing equipment. |
| 8 | An emergency response plan is in place is ready to be used. | • All emergency equipment required by the plan are at the worksite (e.g., electrical safety hooks, insulated gloves, extinguisher for electrical fire etc) • Discuss method of communication with the electrical standby person & rescue team. • Rescue equipment is at the job site • The rescue team: • is available • is aware of specific hazards related to this task • can execute the rescue plan |