| Start-Work Checks ¦ Excavation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## Start Work Authority: Confirm below are in place and verified prior to starting work |  |  |  |  |  |  |
| # Save Your Life Actions All exposed person(s) performing work must confirm each of the action items below |  |  |  | Person(s) Performing Work (initial) | Start-Work Verifier (initial) |  |
| Energy Isolation |  |  |  |  |  |  |
| I have confirmed: |  |  |  |  |  |  |
| 1 | Excavation has been evaluated for energy isolation requirements. Does Excavation work require energy isolation? Yes: No: If yes: Complete Energy Isolation Start-Work Check. If no: Continue to Step 2. |  |  |  |  |  |
| Prior to Excavation activities |  |  |  |  |  |  |
| I have confirmed: |  |  |  |  |  |  |
| 2 | Underground utilities are visibly marked (e.g., pipelines, cables, communications, power). |  |  |  |  |  |
| 3 | Excavation equipment maintains minimum clearances from overhead obstructions |  |  |  |  |  |
| 4 | Excavation area is secured, and barriers are in place to prevent unauthorised access. |  |  |  |  |  |
| 5 | Soil stability has been assessed and controls/safeguards are in place per excavation plan. |  |  |  |  |  |
| 6 | Equipment stability and potential for unplanned movement have been assessed. |  |  |  |  |  |
| Hold Point: Continue if personnel enter excavation |  |  |  |  |  |  |
| 7 | Excavation has been evaluated to determine if it is a confined space. Is Excavation a Confined Space? Yes No If yes: Complete Confined Space Entry Start-Work Checks. If no: Continue to Step 8 |  |  |  |  |  |
| 8 | A plan is in place to protect personnel entering the excavation from: -Cave in. -Hazardous atmosphere; and -Water accumulation. |  |  |  |  |  |
| 9 | Excavations deeper than 1.2 m (4 feet) have access and egress. |  |  |  |  |  |
| ## Stop and seek help if any of the above controls/safeguards are not in place |  |  |  |  |  |  |
| ## Role |  | ## Printed Name | ## Date |  |  |  |
| Start -Work Verifier |  |  |  |  |  |  |

| Save Your Life Actions ¦ Excavation |  |  |
| --- | --- | --- |
| ## Start Work Authority: I will not start work until I confirm… |  |  |
| Actions |  | How to (examples) |
| Energy Isolation |  |  |
| 1 | The excavation has been evaluated for energy isolation requirements. Does Excavation work require energy isolation? Yes: No: If yes: Complete Energy Isolation Start-Work Check. If no: Continue to Step 2. | - • All potential energy sources have been identified, isolated and locked and tagged per isolation plan. - • The underground utility has been drained, flushed, or purged to remove explosive materials or gases. |
| - Prior to Excavation activities |  |  |
| 2 | Underground utilities are visibly marked (e.g., pipelines, cables, communications, power). | - • Local utilities have been consulted about the dig so they can identify their lines (use programmes like Dial Before You - Dig (UK) or Call 811 (US)) - • Underground utilities are visibly identified with flagging or paint - • Depth and width of utilities or structures are known before digging - • Before starting mechanical excavation, actions have been taken to locate and expose underground line/utility and - structures (e.g., probing, hand digging, soft digging, air knifing, hydro-vac) |
| 3 | Excavation equipment maintains minimum clearances from overhead obstructions. | • The exact location, height and voltage of overhead power lines have been identified. • Maintain identified minimum distance between equipment and energy source - • To help with this, use flagging or barries on overhead power lines |
| 4 | Excavation area is secured and barriers are in place to prevent unauthorised access. | - • Excavation area is visibly identified with caution tape, silt fencing or other visual identification. - • Excavation area is secure from unauthorised access. - • No personnel are in line-of-fire hazards (e.g., swing radius of excavator, discharge side of trencher). - • Only essential personnel/crew are in the area where the excavation work is occurring. |
| 5 | Soil stability has been assessed and controls/safeguards are in place per excavation plan. | - • A competent person assessed the soil type to define the excavation safeguards. - • Excavations have a protective system (sloping, shoring, or shielding) in place as applicable - • Storage of excavated material is at least 0.61 m (2 ft) from the edge of excavation - • Ensure stability of adjacent utilities/structures potentially affected by excavation through means of shoring, bracing, and underpinning |
| 6 | Equipment stability and potential for unplanned movement have been assessed. | - • Equipment, load and ground surface have been assessed for stability. - •Verify: • load securing • workplace conditions/travel path • equipment capacity - • Equipment maintains safe distance from the unprotected edges of excavation or trenches to prevent cave ins |
| - Hold Point: Continue if personnel enter excavation |  |  |
| 7 | The excavation has been evaluated to determine if it is a confined space. Is Excavation a Confined Space? Yes: No: If yes: Complete Confined Space Entry Start-Work Checks. If no: Continue to Step 8. | • The excavation has been evaluated to determine if it is a confined space (trench depths greater than or equal to 4 ft (1.2 m) with vertical walls and limited access/egress). • If the excavation is a confined space follow the organization/company's confined space guidance, local regulations and complete the Confined Space Entry Start-Work Check. |
| 8 | A plan is in place to protect personnel entering the excavation from: -Cave in; -Hazardous atmosphere; and -Water accumulation. | - • Excavation inspected by the competent person prior to entry, and as conditions change - • Protective systems are in place and may include: • bracing • shoring • underpinning • benching • Retaining devices or shield systems in place. • Daily inspection are performed to identify hazards and changing conditions. • Initial gas testing is conducted by a Qualified Gas Tester • Required follow up testing frequency is established per the plan • Crew will conduct daily inspection to identify hazards and changing conditions (e.g. contamination, water build up or utilities encountered). |
| 9 | Excavations deeper than 1.2 meters (4 feet) have access and egress. | - • There is a safe means of access and egress when entering an excavation greater than 1.2 meters (4 feet) in depth, up to 6.7 m (25 ft) of lateral travel. Examples are: • ladder • stairways • ramps • sloping for ingress/egress. |