| Start-Work Checks ¦ Man-riding |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## Start Work Authority: Confirm below are in place and verified prior to starting work |  |  |  |  |  |  |
| # Save Your Life Actions All exposed person(s) performing work must confirm each of the action items below |  |  |  | Person(s) Performing Work (initial) | Start-Work Verifier (initial) |  |
| I have confirmed: |  |  |  |  |  |  |
| 1 | Personnel involved with the man-riding operations are qualified to perform their task |  |  |  |  |  |
| 2 | The man-riding equipment is: • certified • inspected • rated for the task. |  |  |  |  |  |
| 3 | All rotating or moving equipment close to man-riding activities are secured to prevent unplanned movement |  |  |  |  |  |
| 4 | Communication methods have been agreed to by the workers |  |  |  |  |  |
| 5 | The tools/materials being used at height are secured |  |  |  |  |  |
| 6 | Barriers and drop/ exclusion zones are in place |  |  |  |  |  |
| 7 | Rescue plan is in place and is ready to be used |  |  |  |  |  |
| ## Stop and seek help if any of the above controls/safeguards are not in place |  |  |  |  |  |  |
| ## Role |  | ## Printed Name | ## Date |  |  |  |
| Start -Work Verifier |  |  |  |  |  |  |

| Save Your Life Actions ¦ Man-riding |  |  |
| --- | --- | --- |
| ## Start Work Authority: I will not start work until I confirm… |  |  |
| Actions |  | How to (examples) |
| 1 | Personnel involved with the manriding operations are qualified to perform their task | • Rider, Winch operator, and Spotter have all been trained and can describe their responsibilities |
| 2 | The man-riding equipment is: • certified • inspected • rated for the task. | • Winch: Certified “Suitable” for Man-riding • Rigging equipment including cable inspected for wear/defects • Harness: Inspected per work at heights requirements • Fall arrest gear (if used): inspected |
| 3 | All rotating or moving equipment close to man-riding activities are secured to prevent unplanned movement | • Hoisting and rotating equipment in the immediate area and at different levels are not moving (e.g., set the brake for the traveling block, lock out tag out) |
| 4 | Communication methods have been agreed to by the workers | • Communication methods agreed to and tested for movement of rider (i.e., hand signals and radios) • Confirm a common set of signals are used by the work crew |
| 5 | The tools/materials being used at height are secured | • Tools used at heights have securing wire/lanyards/tethers • Materials used at height are secured in storage boxes, pouches, bags etc. • Cover openings to lower levels (e.g., grating, gaps etc.) or use debris nets Note: Follow the site/company dropped object prevention program (e.g., Work at Height tool register) |
| 6 | Barriers and drop/ exclusion zones are in place | • Establish drop/exclusion zones • Control access to drop/exclusion zones during work at height (e.g. attendant or physical barriers) |
| 7 | Rescue plan is in place and is ready to be used | • Confirm the work crew has discussed the rescue plan, including: • How to start the rescue response • Location of rescue equipment and responders • Confirm the rescue crew: • is available • is aware of specific hazards related to this task • can execute the rescue plan |